START

Initialise camera, laptop program, ESP32, sensors and motors

Send configuration settings to ESP32

Wait for ESP32 to acknowledge the settings

Wait for the user to press Enter

Identify and store up to two target colours

Lock colours once none are visible for 0.5 s

WHILE the parking task is not complete:

Read camera frames

Detect target colours inside the ROI

Read sensor values from the ESP32

IF camera data or sensor data is unavailable,

or a state timeout is reached:

Send STOP command

Stop the vehicle

Handle the fault

ELSE IF an unparked target colour is detected:

Start the forward timer once

Drive forward while ESP32 wall-following operates

IF the 10 s forward timer finishes:

Send PARK command

ESP32:

Set steering to full lock

Reverse along the arc

Ignore rear sensors during minimum arc time

Compare rear sensors B and B2

Align the vehicle with the wall

Straighten the steering

Reverse straight towards the final position

Stop at the rear-sensor stop distance

Report PARK_DONE

Laptop:

Wait 2 s after PARK_DONE

Record the parked colour

IF another target remains

AND fewer than two bays are parked:

Send UNPARK command

ESP32:

Drive forward

Turn out

Reverse-align

Drive forward again

Report each phase

Report UNPARK_DONE

Wait for UNPARK_DONE

Wait for the old marker to leave the camera view

Resume searching

ELSE:

Send STOP command

Keep the vehicle stopped

Finish the task

ELSE:

Continue driving

Use right-hand sensors to maintain

approximately 105 mm from the wall

END WHILE

STOP